Performance Implications of Hyper-Threading

Yiping Ding, Ethan Bolker, Arjun Kumar

BMC Software, Inc

Intel’s recently introduced Hyper-Threading Technology (HTT) makes a single physical processor appear as two logical processors. Operating systems and applications can schedule processes or threads on those logical processors.  The performance impact of HTT varies, depending on the nature of the applications running on the processors and on how the hardware is configured. This paper illustrates that variation by presenting a set of controlled studies and offers a simple queueing model to interpret the observations. We also examine the issues involved in collecting performance metrics for HTT systems and their implications for modeling.

## Introduction

Computer technology is continually evolving to improve application performance. Each innovation must be evaluated in the field so that practitioners can understand the performance implications for their particular applications, since the actual impact may differ subtly from what was intended.  Intel’s Hyper-Threading Technology (HTT) is one of those recent innovations.  One Intel study shows that HTT sometimes improves performance by 16% to 28% [Marr, et. al.].  But there are times when HTT may degrade application performance. Since all new high-end Intel processors come with HTT it’s there on your new machines whether you know it or not. So it’s important to understand it and to know what it’s doing for you (or against you).

## The Architecture of Hyper-Threading (HTT vs. Dual Processors)

One can improve application performance by optimizing the software (at the application or interpreter/compiler level) or by speeding up the computation in hardware. But a hardware modification may boost performance less than expected if there is a bottleneck somewhere else in the system.  For example, increasing processor clock speed won’t do much if the CPU waits because memory access for instruction fetch is slow. So another route to performance improvement is to remove bottlenecks that cause resources to be idle. Chip architects have been doing this for years, introducing caches, pipelining and instruction prefetch in order to keep the CPU as busy as possible doing useful work.  HTT is another step in that direction.

A hyper-threaded processor duplicates some of the hardware (adding less than 5% to the chip size [Marr, et. al.]) on the path from memory to CPU, but not the CPU itself.

That architecture allows for some parallel processing except at the actual instruction execution phase. That keeps the CPU itself busier, increasing the actual instruction execution rate. You will find technical descriptions in [Marr, et. al.].  HTT makes the chip (IA-32 architecture) appear to the operating system as if it contained two complete physical processors.  That means neither the operating system nor your application need be rewritten to run on a system with HTT, as long as the operating system follows Intel’s straightforward instructions for choosing a (logical) processor on which to schedule work when the system has multiple physical processors.  Of course in order to take advantage of HTT, as is with any multiprocessor system, multiple threads (in one or several processes) must simultaneously want access to a processor [Guptha].  For information on how Microsoft has addressed that issue (and for Windows licensing consequences), see [Borozan]. For Linux HTT support see [Vianney].

Fortunately, you do not need to understand the chip architecture in order to understand how HTT affects the performance of the applications you care about. You can view and analyze HTT as a black box to “feel” its performance impact.  In section 6, we will present a simple queueing model to predict HTT response times based on some baseline measurements. We proceed to that analysis now, starting with the measurements.

- The Meaning of Measurements

Multi-processors present a reporting dilemma to the measurement community.  For example, when both processors on a dual processor machine are active during an interval is the utilization 100% or 200%? To eliminate the ambiguity, we have to specify the number of processors for which the utilization is reported. A performance measurement tool must use one convention consistently and make clear how its output is to be interpreted.  The problem is compounded for a hyper-threaded processor, which is a single physical CPU designed to appear to the operating system and its measurement tools as two separate processors. Since hyper-threading is in principle transparent to the operating system and applications, existing measurement tools need no modifications to run in a hyper-threaded environment. But what will they report? The answer varies. We experimented with four programs: Windows’ Task Manager, BMC PATROL Perceive, and two off-the-shelf tools that make system calls to provide CPU utilization information to a running Java program.  The first is Vladimir Roubtsov’s package [Roubtsov] for querying to discover how many seconds of CPU the Java Virtual Machine (JVM) consumes when running an application. The second is part of the Java Virtual Machine Profiler Interface (JVMPI) [JVMPI] provided by Sun. It collects JVM CPU consumption information on a per thread basis. The table below shows the output for a compute intensive batch workload running on a hyper-threaded machine with one physical processor (two virtual processors) with hyper-threading both on and off..  As you can see, the measurements are contradictory unless properly interpreted.

|  | Hyper-threading |  |  |  |
| --- | --- | --- | --- | --- |
|  | off |  | on |  |
|  | Number of active threads |  |  |  |
| Measurement tool | 1 | 2 | 1 | 2 |
| Task manager | 100 | 100 | 50 | 100 |
| PATROL Perceive | 100 | 100 | 50 | 100 |
| JVMPI | 100 | 100 | 100 | 100 |
| Roubstov | 100 | 100 | 50 | 100 |

Table 1. Reported % CPU Consumption

The two profiling tools do not report utilization directly. We derive that metric using the formula (reported CPU seconds consumed)/(elapsed time). For batch job streams in at most two threads the only problem we see is the factor of two ambiguity illustrated above. But when the work to be done is multithreaded and intermittent more confusing problems occur. We have seen instances in which both tools report utilizations larger than 100%. We have not looked at the code to understand why, but suspect that each process (or thread) is thought by the tool to be consuming CPU when it is running on one of the logical processors. But when both logical processors are busy (in that sense) only one of them can be executing an instruction, since they share the single physical processor. Thus both will be charged for the same real CPU time, leading to over counting that will be clearly visible if the total CPU seconds the tool thinks the process consumed exceeds the elapsed time. Even when it’s not, the problem persists. The busier the system the greater the probability that the tools will report that it takes more seconds of (purportedly real) CPU to do the same amount of work.

In order for a measurement tool to report reliable and useful information it should know the map between logical and physical processors. Intel provides an assembly level API [HTT-API] for accessing that information.  For .Net / Windows 2003, Microsoft will provide this information in its performance registry as well.

- Batch Processing

The application we ran in order to investigate what the measurement tools report does more than exercise those tools. It is designed and instrumented to study processor performance under a variety of configurations.  The first test we performed was to set up a number of computation intensive batch job streams, each running in a separate (Java) thread, and measure system performance – in this case the statistics of interest are the number of threads, the throughput, and the CPU utilization. We expect the last of these to be 100% of all requested processors, suitably interpreted. Table 1 reports the throughputs for four configurations: single and dual (physical) processors with hyper-threading both enabled (“on”) and disabled (“off”). In each case the utilizations (which we have not included) show that all the virtual processors for which there is a ready thread are constantly busy.

|  | Physical Processors |  |  |  |
| --- | --- | --- | --- | --- |
|  | 1 Processor 2400 MHz |  | 2 Processors 2000 MHz each |  |
|  | Hyper-threading |  |  |  |
| Threads | off | on | off | on |
| 1 | 365 | 389 | 317 | 315 |
| 2 | 369 | 439 | 635 | 597 |
| 3 | 372 | 440 | 636 | 664 |
| 4 | 372 | 439 | 634 | 795 |
| 5 | 373 | 443 | 636 | 803 |
| 6 | 374 | 442 | 639 | 929 |
| 7 | 371 | 443 | 634 | 889 |
| 8 | 376 | 442 | 634 | 881 |

Table 2. Batch throughput

Figure 1. Batch throughput curves

We can draw some interesting conclusions from these data.   With hyper-threading on or off the single processor system behaves just as we expect it to. The batch throughput is essentially independent of the number of threads.  It is slightly larger with hyper-threading on even for single threaded work, probably because the other background work on the system (like data collection tools) can use the second logical processor and thus interfere less with the batch job stream.

The two-processor system (four virtual processors) can run up to four threads simultaneously. With hyper-threading off the throughput per processor is essentially proportional to processor clock speed when compared to the single processor system.  When the batch job stream is single threaded it keeps just one of the real processors busy. The second is free to do system work, whether or not hyper-threading is enabled, so batch throughput is the same in both cases. With two batch threads the system is smart enough to schedule work on both physical processors and the throughput nearly doubles. It doesn’t quite double, probably because the system must still use resources to run itself. As the number of threads increases to three and then to four the throughput continues to increase when hyper-threading is enabled. Here we see performance improvement in the batch job streams themselves, due to hyper-threading, which introduces some low level parallelism in first one and then both processors.

- Transaction Processing

In our next set of experiments we configured our benchmark driver to simulate the computation part of a transaction processing or web server workload. At random times requests for a random amount of CPU service arrive at the system. Each request is assigned a thread. Thus the number of active threads varies over time. The larger the arrival rate and average service time the greater the utilization and the higher the probability that there will be many jobs (and hence many threads) active simultaneously. We studied how response time varies with increasing load.

We set out to validate our benchmark software by studying how measured response times for a single processor (with hyper-threading off) matched those predicted by standard queueing theory: when processor utilization is U (0 < U < 1) it should take 1/(1-U) seconds of real (wall clock) time (on average) to accumulate one second of processor time. Thus the response time for an average 1 second job should be 1/(1-U) seconds. Table 3 and Figure 2 compare measured and predicted response times.

In the table utilizations and throughputs are nearly the same since the average job size is close to one second.

| Utilization | Throughput (jobs/sec) | Response time |  |
| --- | --- | --- | --- |
|  |  | measured | predicted |
| 0.183 | 0.196 | 1.11 | 1.14 |
| 0.268 | 0.288 | 1.21 | 1.25 |
| 0.347 | 0.374 | 1.34 | 1.40 |
| 0.434 | 0.465 | 1.53 | 1.61 |
| 0.508 | 0.546 | 1.71 | 1.83 |
| 0.586 | 0.621 | 1.96 | 2.19 |
| 0.657 | 0.696 | 2.28 | 2.60 |
| 0.705 | 0.733 | 2.57 | 3.06 |
| 0.743 | 0.770 | 2.77 | 3.50 |
| 0.768 | 0.799 | 2.98 | 3.84 |

Table 3. Response times for one-second job (single processor, hyper-threading off, simple M/M/1 model for prediction)

Figure2 Response time curves for one-second job (single processor, hyper-threading off, simple M/M/1 model for prediction)

We found measured response time systematically smaller than predicted response time. The discrepancy increases with increasing utilization. We seem to see less randomness than the exponential distributions the driver is programmed to produce. We measured the actual coefficients of variation for these statistics and discovered that the interarrival times are in fact more regular than they should be, i.e., the coefficient of variation is less than 1. But the anomaly persists even after we correct for that phenomenon by modifying the naïve 1/(1-U) prediction to take into account the actual coefficient of variation of the job interarrival times.  That suggests that a different queueing model may be needed to reflect the true execution structure of a processor with hyper-threading technology even when hyper-threading is disabled. We will present such a model in Section 7.

5. Hyper-threading On vs. Hyper-threading Off

We do not need a queueing theory model just to compare system performance with hyper-threading on and off. We need only run our benchmark with hyper-threading enabled in order to see what happens. Figure 3 illustrates the results. (The response time data when hyper-threading is on appear in a later table).

Figure 3. Response time curves for one-second job (hyper-threading on and off)

As expected, hyper-threading improves response time for given throughput, and the improvement increases as the throughput increases.

The fact that response times for given throughputs are significantly smaller when hyper-threading is on indicates significant benefit for transaction processing workloads.  The maximum throughput possible while still meeting response time service level objectives is larger when hyper-threading is on.

It’s important to use throughput and not utilization as the independent variable here, since we’ve seen that utilization measurements are suspect when hyper-threading is enabled. Fortunately, the important metric in a transaction processing system isn’t the utilization, it’s the response time that corresponds to a given throughput. You need not care how busy your system is – what really matters is how response time varies with system load as defined by useful work done – that is, by throughput.

6.  A Queueing Theory Model for a Hyper-Threaded Processor

In order to understand the response times when hyper-threading is enabled we developed a queueing theory model for the hyper-threaded architecture described in Section 1 when there is just one physical processor. From the application’s point of view each job visits one of the two logical processors offered to it by the operating system. On the chip, each instruction execution goes through two phases. First one instance of the duplicated part of the hardware does preparatory work, looking up data and instructions in the cache or getting them from memory. Then the single real CPU executes the instruction. We model this situation with the three queues shown in Figure 4. Tasks arrive at the system at the rate of λ per second and go to one of the two preparatory queues with probabilities p1 and p2, respectively. Since each job goes someplace, p1+p2=1. We will assume that load balancing works as it should, so that p1=p2 = 1/2.

p1                                λ

λ

p2

Figure 4. A simple queueing model for hyper-threading.  The solid circle represents the single physical processor; the open circles the places where work is done prior to instruction execution.

Suppose the service time at the preparatory queue is s1 seconds/job and the service time at the real CPU is s2seconds/job and that the job interarrival times and service times are exponentially distributed. We then estimate the mean response time using an open model:

R=p1s11−p1λs1+p2s11−p2λs1+s21−λs2,

=s11−λ2s1+s21−λs2,                               (1)

since we have assumed p1=p2 = 1/2.

In order to use this model to predict the response times we measured in our experiments, we need values for λ, s1 and s2. The first of these we know. It’s an input parameter. We can measure the average total service time for each job, seen from the outside, but that does not tell us the internal service times. All we can be sure of (when the system is not saturated) is

p1λs1<1, and

p2λs1<1, and

λs2<1.

In order to test the model and to understand hyper-threading architecture, we found the values of s1 and s2 that best fit the measured data. Those values turned out to be

s1 =  0.13, s2 = 0.81.

That suggests that about 15% of the work occurs in the parallel preparatory phase of the computation, while instruction execution consumes about 65%.

Table 4 and Figure 5 show the results of our experiment and analysis.

The fact that we can fit the measured data so well with reasonable parameter values suggests that the model is sound.

| Utilization (nominal) | Throughput (jobs/sec) | Response time |  |
| --- | --- | --- | --- |
|  |  | measured | predicted |
| 0.110 | 0.200 | 1.12 | 1.10 |
| 0.174 | 0.298 | 1.24 | 1.20 |
| 0.237 | 0.387 | 1.36 | 1.32 |
| 0.313 | 0.486 | 1.55 | 1.48 |
| 0.388 | 0.578 | 1.73 | 1.67 |
| 0.468 | 0.667 | 2.02 | 1.91 |
| 0.545 | 0.754 | 2.35 | 2.25 |
| 0.578 | 0.795 | 2.41 | 2.44 |
| 0.617 | 0.842 | 2.56 | 2.72 |
| 0.658 | 0.887 | 2.74 | 3.07 |

Table 4. Response times for one-second job (single processor, hyper-threading on, predictions from model in Figure 4)

Figure 5. Response time curves for one-second job (single processor, hyper-threading on, predictions from model in Figure 4)

7.  A Tandem Queue Model for a Processor with Hyper-Threading off

In Section 4 we used a simple M/M/1 model to try to predict the response times of jobs running on a hyper-threaded processor for which hyper-threading was disabled. The theoretical and measured results did not match.  We need a better model.  Fortunately, the one we developed for the system when hyper-threading is enabled works here too. When hyper-threading is disabled we assume that jobs still visit the chip hardware that does preparatory work before moving to the execution phase. The difference is that there is just one queue and one server in the preparatory phase.

That is the model in Figure 4 with p1= 1 and p2 = 0. We redraw it in Figure 6.

λ                          λ

Figure 6. A tandem queue model for hyper-threading disabled.  The solid circle represents the single physical processor; the open circle the place preparatory work is done prior to instruction execution.

The response time, under the same statistical assumptions, is

R=s11−λs1+s21−λs2,                                (2)

where s1 is the mean service time for the preparatory work, which is non-zero even when the hyper-threading is disabled, and s2 is the mean service time in the execution phase.

As in the previous section, we found the values of s1 and s2 that best predict the measured data:

s1  = 0.045, s2 = 0.878.

Note that when hyper-threading is disabled the nominal service time for the preparation work is much smaller than that when hyper-threading is on.

Table 5 and Figure 7 show the measured and predicted response times using the tandem queueing model.  These match remarkably well, suggesting that this is in fact a good model for the chip architecture.

| Utilization | Throughput (jobs/sec) | Response time |  |
| --- | --- | --- | --- |
|  |  | measured | predicted |
| 0.183 | 0.196 | 1.11 | 1.11 |
| 0.268 | 0.288 | 1.21 | 1.22 |
| 0.347 | 0.374 | 1.34 | 1.35 |
| 0.434 | 0.465 | 1.53 | 1.53 |
| 0.508 | 0.546 | 1.71 | 1.73 |
| 0.586 | 0.621 | 1.96 | 1.98 |
| 0.657 | 0.696 | 2.28 | 2.31 |
| 0.705 | 0.733 | 2.57 | 2.51 |
| 0.743 | 0.770 | 2.77 | 2.75 |
| 0.768 | 0.799 | 2.98 | 2.99 |

Table 5.  Response times for one-second job (single processor, hyper-threading off, predictions from tandem queue model in Figure 6)

Figure 7. Response time curves for one-second job (single processor, hyper-threading off, predictions from tandem queue model in Figure 6)

8. Future Work

Our work so far shows that it is possible to measure and model application performance in a hyper-threaded environment. Our plans now are to

- Explore how the model parameters s1 and s2depend on the nature of the instructions being executed.

- Find ways to compute or estimate s1 and s2from first principles rather than by finding the best fit to measured data.

- Test our general queueing model on systems with multiple physical processors each of which is hyper-threaded.

9. Summary

Hyper-threading is complex, interesting and ubiquitous. Until measurement tools have caught up to it, you must treat what they say with some caution and insight.

In general, Intel’s claims for the performance benefits are believable, but you must be a little careful. Single threaded applications may run better with hyper-threading disabled. Measure and act accordingly.

It is possible to construct relatively simple queueing theory models for hyper-threaded processors.

10. References

[Borozan] John Borozan, Microsoft Windows-Based Servers and Intel Hyper-Threading Technology, Microsoft Windows Technical Article, April 2002, http://www.microsoft.com/windows2000/docs/hyperthreading.doc

[Friedman] Mark Friedman. Hyper-threading - Two for the price of one? , Measure IT, March 1, 2003

http://www.cmg.org/measureit/issues/mit01/m_1_1.html

[Guptha] Soumya Guptha, “Multithreaded Programming in a Microsoft Win32 Environment,” http://cedar.intel.com/media/pdf/xeon_mp_32.pdf

[HTT-API] http://cedar.intel.com/media/training/detect_ht_dt_v1/tutorial/index.htm

## [JVMPI] Java Virtual Machine Profiler Interface (JVMPI) http://java.sun.com/products/jdk/1.2/docs/guide/jvmpi/jvmpi.html

[Marr, et. al.] Deborah Marr, Frank Binns, David Hill, Glenn Hinton, David Koufaty, J. Miller, Michael Upton, “Hyper-Threading Technology Architecture and Micro architecture,” Intel Technology Journal (Hyper-Threading Technology),

Volume 06 Issue 01, February 14, 2002.

http://www.intel.com/technology/itj/2002/volume06issue01/vol6iss1_hyper_threading_technology.pdf

[Roubtsov] Vladimir Roubtsov, Profiling CPU usage from within a Java application,

http://www.javaworld.com/javaworld/javaqa/2002-11/01-qa-1108-cpu.html

[Vianney] Duc Vianney, Hyper-Threading speeds Linux, IBM DeveloperWorks, http://www-106.ibm.com/developerworks/linux/library/l-htl/

Appendix. Experimental Setup

System configuration. The single processor machine runs a 2400 MHz Intel PowerEdge CPU. The dual processor machine runs two similar 2000 MHz CPUs. The operating system on both machines is Windows 2000, running standard utilities (Task Manager, BMC Collect, virus scan) and Java 1.4.

Benchmark software. We wrote and instrumented a multithreaded Java application configurable to generate various kinds of compute intensive workloads. In our experiments the computation consisted of multiple invocations of method .log in the Java Math class; we varied the way in which the system was called upon to compute those logarithms.

- Batch, with a specifiable number of job streams each running in its own thread. Each stream presents the system with a sequence of identical jobs. Each job takes about 100 seconds on the single processor machine with hyper-threading disabled.

- Open, with specified arrival rates and service times, each job running in its own thread. In these experiments the jobs are shorter and their actual sizes are determined randomly. On average, each calls for one second of computation on the single processor machine. Interarrival times and service demands are exponentially distributed.

- Closed, with a specifiable number of job streams, specified think times and job service times. Jobs are as in the open experiment. Think times and service demands are exponentially distributed. This paper does not deal with the results for closed job streams.

Ethan Bolker is also Professor of Computer Science at the University of Massachusetts, Boston.

Some specialized applications like compilers and video games might benefit from HTT aware coding.

Since hyper-threading is set in the BIOS you must reboot to enable or disable it.